Ex. No: 09                            CAPTURE PHOTOS ON MOBILE PHONE’S CAMERA

Date: 27-01-26                                       CONNECTED TO RASPBERRY PI

AIM:

To capture photos using a mobile phone camera connected to raspberry pi where the photo shots are clicked by commands sent to raspberry pi to mobile phone and photo to be stored in the raspberry pi.

APPARATUS REQUIRED:

- Smartphone with camera (poco x4 pro)

- IP Webcam mobile application (available in playstore)

- Raspberry pi model B v1.2

- USB cable (for power supply to pi)

IMPLEMENTATION:

There are different ways to make connection between phone camera and raspberry pi :

- gphoto2 tool

- The "ADB" Method (Android Debug Bridge)

- IP Webcam mobile application

1. Why did using gphoto2 fail ?

gphoto2 is a free open-source command-line tool and library (libgphoto2) for Linux/Unix systems used to control, configure, and download images from over 1,000 digital cameras. It allows users to remotely trigger shutters, change camera settings (ISO, aperture), and automate shooting via USB.

Plugging the usb from raspberry pi to phone using usb data cable, in phone android gives multiple options to use the usb for : to get charge, FTP, PTP (Picture Transfer Protocol).

In raspberry pi :

It failed because the OS in the smartphone doesn’t allow providing access to the raspberry pi to take control of the shutter and capture the image. Found that only works for camera sensors.

2. why did using ADB Method (Android Debug Bridge) not efficient ?

This tool allows the Pi to send commands to the phone, like "Open Camera" and "Click Shutter." ADB tools have set of tools used for the development of mobile application by connecting the physical smart phone device to the development environment ie., a computer.

In mobile:

Enable developer options (Go to Settings > About Phone > Tap "Build Number" 7 times). Go to Developer Options and enable USB Debugging. Connect to Pi via USB.

In raspberry pi:

It works but it is not efficient. Because any phone connected to the pi needs developer options enabled and script overhead. It is not a plug and play solution for connecting any android phone.

3. Why IP Webcam mobile application is the best solution :

IP Webcam is an Android app that turns the phone into a mini web server.

The Phone creates a website (e.g., http://192.168.1.15:8080) that displays its camera view.

The Raspberry Pi "visits" that website over Wi-Fi and downloads the image, exactly like downloading a file from the internet.

Install IP Webcam android app from play store, have the mobile phone connected to same wifi as raspberry pi, open the app and start the server.

In raspberry pi:

It is the best and optimal way to capture a photo using a mobile phone camera and store it in raspberry pi using a single command in raspberry pi.